Table 3.  Current and projected US gross annual energy supply from various renewable energy technologies based on the thermal equivalent and required land area.

Current (2000)		     Projected (2050)

# Energy technology		Quads		Million ha	Quads		Million ha

# Biomass			3.6a		  75b		    5		102b

# Hydroelectric			3.9a		  26d		    5		  33

# Geothermal			0.3a		    0.4c		    1.2		    1

# Solar thermal		       <0.04a	             <0.01	              10		  11

# Photovoltaics 		       <0.04c		 <0.01	              11		    3

Wind power		         0.05c		   0.5	                7		    8

Biogas			      <0.001c		 <0.001	     0.5		     0.01

Passive solar		        0.3e		    0		     6		    1

TOTAL		        8.2		102	               45.7		159____

a  (DOE/EIA 1998).

b  This is the equivalent land area required to produce 3t/ha plus the energy required for harvesting and transport.

c  Estimated.

d  Total area based on an average of 75,000 ha per reservoir area per 1 billion kWh/yr produced.

e  (Pimentel et al. 1994).